MIMIC®

TECHNICAL SPECIFICATION DATASHEET

BUZZER CUM FLASHER  •  MODEL BZ1  •  24 V DC

| MODEL BZ1 | OPERATING VOLTAGE 24 V DC | ORDERING CODE BZ1 |
| --- | --- | --- |

PRODUCT DESCRIPTION

The MIMIC BZ1 Buzzer Cum Flasher is a compact industrial audio-visual signalling device that combines an intermittent electronic buzzer and flashing red LED in one unit. It provides simultaneous audible and visual warning while saving control-panel space. The unit is designed for control panels, distribution boards, industrial automation, machinery, process monitoring and safety-warning applications.

KEY FEATURES

• Integrated buzzer & flashing LED indicator in one unit; saves panel space

• 90 dB audible alarm at 1 meter with bright LED flash

• 22.5 mm standard panel cutout for easy installation

• Compact panel-mounting design

• IP54 rated front for dust & water protection

• Low power consumption and reliable solid-state design

• Maintenance-free, long-life LED

• Continuous-duty rated

• Reverse-polarity protected

• PBT flame-retardant body, UL94 V-0; polycarbonate glowing cap

ELECTRICAL SPECIFICATIONS

| PARAMETER | SPECIFICATION |
| --- | --- |
| Brand | MIMIC |
| Model | BZ1 |
| Operating Voltage | 24 V DC |
| Current | 16 mA |
| Voltage Tolerance | ±20% |
| Power Consumption | < 5 VA / < 50 mA |
| Operating Mode | Buzzer Beeping & Flashing at a Time |
| Sound Output | 90 dB at 1 meter |
| Tone Type | Intermittent Beeping |
| Flash Rate | 60 flashes / minute (1 flash per second) |
| LED Colour | Red |
| Duty Cycle | Continuous Rated |
| Protection | Reverse Polarity Protected |

MECHANICAL SPECIFICATIONS

| PARAMETER | SPECIFICATION |
| --- | --- |
| Enclosure | PBT, Flame Retardant Body – UL94 V-0 |
| Cap | Polycarbonate, Glowing Cap |
| Mounting | Panel Mounting, 22.5 mm Cutout |
| Overall Size | 32 × 42 × 80 mm |
| Behind Panel | 60 mm |
| Front Protection | IP54 |
| Weight – Main Unit | 56 g |
| Weight – With Carton | 64 g |

BZ1  •  TECHNICAL, APPLICATION & ORDERING DATA

PACKAGING SPECIFICATIONS

| PARAMETER | SPECIFICATION |
| --- | --- |
| Carton Size | 44 (L) × 34 (W) × 86 cm |
| Units in One Main Carton | 35 units |
| Main Carton Size | 225 (L) × 190 (W) × 125 mm |
| Main Carton Weight | 2 kg 300 g with 35 units |

CONNECTION DETAILS

| PARAMETER | SPECIFICATION |
| --- | --- |
| Connection | +12 V, 0 V to Pin 1 & 2 respectively |
| Connection Wires | 1.5 to 2.5 mm² × 2 Nos |

SYSTEM COMPARISON

| OLD SYSTEM | MIMIC BZ1 |
| --- | --- |
| Separate buzzer + separate lamp + flasher relay | All-in-one buzzer + flashing LED unit |

APPLICATIONS

• Control Panels & Distribution Boards

• Industrial Automation & Machinery

• Process Control & Monitoring Systems

• Alarm & Safety Warning Systems

• HVAC & Building Management

• Material Handling & Conveyors

ORDERING DETAILS

| PARAMETER | SPECIFICATION |
| --- | --- |
| Ordering Code | BZ1 |
| Ordering Name | Mimic Buzzer Cum Flasher, Model: BZ1, Voltage: 24V DC |
| Ordering Email ID | mimic.sales@gmail.com |
| Ordering Phone (WhatsApp) | 9321155319 |
| Technical Inquiry | 9321252022 |

WARRANTY

12 MONTHS

ORDERING ADDRESS

Steam Airetric Controls
Plot No. A/34, Additional Ambernath MIDC, Anand Nagar, Ambernath (E), Dist. Thane,
Ambernath, Maharashtra, India – 421506